Course:	CMCE1104 Statics

Project #2: 	Trusses: Method of Joints and Sections

Purpose: 	A roof truss at the Home Depot® is to be designed.  Each student is to determine the loading in each chord of the Pratt Truss using the Method of Joints and Sections and to check your calculations using computer software.

Directions: 	Each student is to complete and submit the requested information in the following order:

- Cover page is to be typed and include: your name, project name, project number, case number, date due and date submitted. (0.5 points)

- Exploded Free Body Diagram (FBD) showing all reactions and bar forces.  Label bar forces in tension (T) or compression (C). (0.5 points)

- Calculate the reactions at points 1 and 8.  Assume that the truss is simply supported with a pin at point 1 and a roller at point 8.  Check using an independent equation. (1 point)

- Calculate the bar forces. The calculations should be in a logical, organized format.  A FBD must be drawn for each Joint and Section analyzed with a maximum of 2 Joints per page and a maximum of 1 Section per page shown.  The following sections and joints must be analyzed for full credit:

  - Method of Joints: Show calculations for joints 1, 2, 5, 6, 8 (ΣFx = 0 and ΣFy = 0) (4 points)

  - Method of Sections: Show calculations for sections 1-1 and 2-2 (2 points)

  - Check: with joint 4 (1 point)

- MDSolids Computer Solution Printout.  Using the computer program "MDSolids,” determine the individual bar forces for each truss member. The computer program can be downloaded for FREE at http://www.mdsolids.com/download.htm. The version of MDSolids that you download from this site may be used free-of-charge for a 30-day evaluation period. (1 point)

Due Date:	April 23, 2013

TRUSS PROJECT

| CASE NO. | TRUSS HEIGHT "H" | PANEL WIDTH "L" | VERTICAL LOAD (kips) | LOCATION OF APPLIED LOAD (Joint) |
| --- | --- | --- | --- | --- |
| 1 | 8'-0" | 6'-0" | 10 | 2 &7 |
| 2 | 9'-0" | 7'-0" | 10 | 3 & 6 |
| 3 | 10'-0" | 8'-0" | 10 | 4 & 3 |
| 4 | 11'-0" | 9'-0" | 10 | 4 & 7 |
| 5 | 12'-0" | 10'-0" | 10 | 5 & 6 |
| 6 | 8'-0" | 6'-0" | 20 | 4 & 6 |
| 7 | 9'-0" | 7'-0" | 20 | 2 & 5 |
| 8 | 10'-0" | 8'-0" | 20 | 4 & 3 |
| 9 | 11'-0" | 9'-0" | 20 | 4 & 7 |
| 10 | 12'-0" | 10'-0" 1 | 20 2 | 5 & 6 |

1

2