Project 1

Team 1:  Mamadou Bah, Arie Meiman, Daniel Edwards

- A 690λ- long lossless Z0=75Ω T-line is terminated in in a load ZL=15+j67 Ω. Calculate and use the Smith Chart  to find:

ZL (normalized)= ZL / Z0  = 15+j67 Ω / 75Ω = 0.2+j0.9 Ω

Length=690 λ/0.5(full rotations) = 1380 full rotations,** the length is equal to zero**

- ΓL=(ZL – Z0)/( ZL+ Z0)=( 15+j67 Ω - 75Ω)/( 15+j67 Ω + 75Ω)= (-60+j67)/(90+j67)

= 89.94 /_ 132° / 112.2/_ 36.6° = 0.804 /_ 95.4°

b) VSWR=[1+Γ]/[1-Γ]=[1-0.804]/[1+0.804]=9.2

c) Zin  = Z0*[ ZL+ j*Z0 *tan(βl)]/ [ Z0+ j*ZL *tan(βl)]= ZL  ***Since length is equal to zero(l=0 λ)

- The input impedance for a 100Ω lossless T-line of length 1.162λ is measured as 12+j42 Ω. Determine the load impedance.

- We normalize the load impedance: Zin’= 0.12+j0.42 Ω

- We moved along the perimeter counter-clockwise going towards the load for a total length of 0.162λ.

- We found ZL on the Γ circle, ZL=1.5-j0.7 Ω

- Then we multiply the normalized ZL by the Z0 , ZL=ZL* Z0 =150-j70 Ω

- Design an open ended shunt-stub matching network to match a load ZL =70+j110 Ω to a 50 Ω  impedance T-line. Chose the solution that minimizes the length of the through line.

- We normalize the load impedance: ZL’= 1.4+j2.2 Ω

- we found the normalized admittance by passing through the center of the circle and the intersection of the line and  is

- we moved along the perimeter clockwise going to the generator until we intersect with the 1+j*b (b=1.9)

- we found 0–j*b and draw a line that passes to center of the circle;

- the intersection of the line and the outer circle of the smith chart gives us length and distance;

- the length is found by subtracting from this distance 0.25 λ, l=0.327 λ -0.25 λ=0.077 λ

- the distance that we need to travel is d=0.185 λ+0.05 λ=0.19 λ

- A load impedance ZL=25+j90 Ω is to be matched to a 50 Ω line using a shorted shunt stub tuner. Find the solution that minimizes the length of the shorted stub

- We normalize the load impedance: ZL’= 0.5+j1.8 Ω

- we found the normalized admittance by passing through the center of the circle and the intersection of the line and  is

- we moved along the perimeter clockwise going to the generator until we intersect with the 1+j*b (b=2.6)

- we found 0–j*b and draw a line that passes to center of the circle;

- the intersection of the line and the outer circle of the smith chart gives us length and distance;

- the length is found by subtracting from this distance 0.25 λ, l=0.308 λ -0.25 λ=0.058 λ

- the distance that we need to travel is d=0.198 λ+0.078 λ=0.276 λ

- A 50 Ω  line is terminated in a pair of parallel load impedances of 50+j100 Ω and 50 –j100 Ω. Determine the total load admittance and impedance seen by the line.

- To find total load impedance we calculate ZL(T)=(50+j100)*(50-j100)/(50+j100+50-j100)=250 Ω

- Admittance is simply the inverse of impedance, YL=1/ZL=1/250=0.004S

- To find normalized load we simply divide by 50 , ZL(T)/50=5 Ω

- Consider the figure with the following values VS=10V, RS=30Ω, R0=50Ω, up=0.666c, RL=150Ω, and l=10cm. Plot out to 2ns

- the voltage at the source end, Vs=10V

- the voltage at the middle,

- the voltage at the load end of the T-line.