Bibin Koshy									Professor Viviana

TCET 2220

12/16/13

Homework #5

5-1. A 50-Ω lossless line is terminated in Z2 =15-j20 Ω.

- Plot the normalized impedance on Smith chart.

- Using the Smith chart, determine the corresponding normalized admittance y2.

- Using the result of (b), determine the actual admittance Y2.

5-2.  A 50-Ω lossless line is terminated in Y2 =.001 +j0.02S.

(a)	Plot the normalized admittance on Smith chart.

(b)	Using the Smith chart, determine the corresponding normalized impedance Z2.

(c)	Using the result of (b), determine the actual impedance Z2.

5-3. For the system of Problem 5-1, determine the VSWR and the reflection coefficient.

5-4. For the system of Problem 5-2, determine the VSWR and the reflection coefficient.

5-5. A 50-Ω lossless line is terminated in a real load impedance Z2 =R2=150Ω. Using a Smith chart, determine the input impedance for each of the following line lengths:

- 0.15 λ

- 0.25 λ

- 0.35 λ

- 0.45 λ

5-6. A 50-Ω lossless line is terminated in a real load admittance Y2 =0.01S. Using a Smith chart, determine the input admittance for each of the following line lengths:

- 0.125 λ

- 0.25 λ

- 0.375 λ

- 0.50 λ

5-7. A 50-Ω lossless line is terminated in a real load impedance Z2=75+j100 Ω.

- Using a Smith chart, determine the input impedance for a line of length 0.2 λ.

- Determine the VSWR and the reflection coefficient at the load.

5-8. A 50-Ω lossless line is terminated in a real load impedance Z2=150-j240 Ω.

- Using a Smith chart, determine the input impedance for a line of length 0.3 λ.

- Determine the VSWR and the reflection coefficient at the load.