Table of Contents

Problem # 1	2

Part 1: Finding the reflection coefficient, the Voltage Standing Wave Ratio, the input impedance by calculation.	2

Part 2:  Finding the reflection coefficient, the Voltage Standing Wave Ratio, the input impedance using the Smith Chart.	3

Problem # 2	4

Project 1

Team 1:  Mamadou Bah, Arie Meiman, Daniel Edwards

## Problem # 1

A 690λ- long lossless Z0=75Ω T-line is terminated in in a load ZL=15+j67 Ω. Calculate and use the Smith Chart  to find:

- ΓL;

- VSWR;

- Zin

Solution:  This problem is divided into two parts:

### Part 1:

Calculation part

Zl=15+i*67;               % Load impedance
Zo=75;                    % Characteristic impedance of the transmission Line
% The reflection reflexion
Lamda=(Zl-Zo)/(Zl+Zo);    % The reflexion coefficent at the load
mag=abs(Lamda); teta=angle(Lamda);
disp('*********************************************')
fprintf('The magnitude of the reflexion coefficient is :%f\n and and the phase in degree is : %f\n',mag,teta*180/pi)
% The voltage standing wave ratio
disp('*********************************************')
VSWR=(1+mag)/(1-mag);
fprintf('The voltage standing wave ration is equal to : %f\n',VSWR)
disp('*********************************************')
% The input Impedance
% tan(beta*lenght)=0.275;
Zin=Zo*(Zl+i*Zo*(0.275))/(Zo+Zl*i*(0.275));
fprintf('The input impedance is equal to : %f\n',Zin)
disp('**********************************************')

*********************************************
The magnitude of the reflexion coefficient is :0.801589
 and and the phase in degree is : 95.179522
*********************************************
The voltage standing wave ration is equal to : 9.080091
*********************************************
The input impedance is equal to : 28.204791
**********************************************

### Part 2:

- The reflection coefficient:

- First we have to normalize the load impedance:

- We find the intersection of the real circle (x= 0.2) circle and the imaginary circle (Im=0.8933)

- The Voltage Standing Wave Ratio: from the smith chart VSWR= 3.2

- The input impedance:

## Problem # 2

The input impedance for a 100Ω lossless T-line of length 1.162λ is measured as 12+j42 Ω. Determine the load impedance.

Solution:

t=0.12812;
Z0=100; Zin=12+j*42;    %t=tan(beta*lengh)
Zl=(Z0)*(Zin-j*Z0*t)/(Z0-j*Zin*t);
disp('*********************************************')
disp('The load impedance in ohm is :')
disp(Zl)
disp('*********************************************')

*********************************************
The load impedance in ohm is : 10.9808 +27.8578i
*********************************************

## Problem # 3

a) Design an open ended shunt-stub matching network to match a load ZL =70+j110 Ω to a 50 Ω impedance T-line. Choose the solution that minimizes the length of the through line.

- Now suppose the load turns out to be ZL =40+j100 Ω. Determine the reflection coefficient seen looking into the matching network.

Solution:

- We normalize the load impedance:

- we found the normalized admittance by joining the drawing a line tha start from the intersection of 1.4 j*2.2 and passing by the center of the circle and the intersection of the line and  is

- we moved along the perimeter clockwise going to the generator until we intersect with the 1+j*b

- we found –b and draw a line that passes to center of the circle;

- the intersection of the line and the outer circle of the smith chart gives us and distane;

- the length is found by subtracting this distance to with give us

- the distance that we need to travel is found by subtraction

## Problem # 4

A load impedance ZL=25+j90 Ω is to be matched to a 50 Ω line using a shorted shunt stub tuner. Find the solution that minimizes the length of the shorted stub

Solution

- We normalize the load impedance:

- we found the normalized admittance by joining the drawing a line tha start from the intersection of 0.1433 j*0.515 and passing by the center of the circle and the intersection of the line and  is

- we moved along the perimeter clockwise going to the generator until we intersect with the 1+j*b

- we found –b and draw a line that passes to center of the circle;

- the intersection of the line and the outer circle of the smith chart gives us and distane;

- the length is found by subtracting this distance to with give us

- the distance that we need to travel is found by subtraction

## Problem # 5

- A 50 Ω line is terminated in a pair of parallel load impedances of 50+j100 Ω and 50 –j100 Ω. Determine the total load admittance and impedance seen by the line.

Solution:

zl2=50+j*100;
zl1=50-j*100;
zl=(zl2*zl1)/(zl2+zl1); yl=1/zl;
disp('*********************************************')
disp('The total load impedance in ohm is ')
disp(zl)
disp('*********************************************')
disp('The total load admittance  in siemens is ')
disp(yl)
disp('*********************************************')

*********************************************
The total load impedance in ohm is 
   125
*********************************************
The total load admittance  in siemens is 
    0.0080
*********************************************

## Problem # 6

Consider the figure with the following values VS=10V, RS=30Ω, R0=50Ω, up=0.666c, RL=150Ω, and l=10cm. Plot out to 2ns

- the voltage at the source end,

- the voltage at the middle,

- The voltage at the load end of the T-line.

Solution