Team A: Jose Chiriboga Alcivar, Juan Torres, Nestor Lam
Extra Credit

- A 690λ- long lossless Z0 = 75 Ω T-line is terminated in in a load ZL = 15 + j67 Ω. Calculate and use the Smith Chart  to find:

ZL (normalized)= ZlZo  = 15+j67 Ω75Ω = 0.2+j0.9 Ω

(a)

ΓL = (ZL – Z0)( ZL+ Z0) = ( 15+j67 Ω-75Ω)( 15+j67 +75Ω)= (-60+j67)(90+j67)=89.94 /_ 132° 112.2/_ 36.6°

=0.804/_95.4°

b)

VSWR=1+Γ1-Γ =1-0.8041+0.804=9.2

- The input impedance for a 100Ω lossless T-line of length 1.162λ is measured as 12 + j42 Ω. Determine the load impedance.

- We normalize the load impedance:

Zin= 12+j42 Ω100Ω=0.12+j0.42 Ω

- We moved along the perimeter counter-clockwise going towards the load for a total length of 1.162λ.

- We found ZL on the Γ circle,

ZL=1.5 - j0.7 Ω

- Then we multiply the normalized ZL by the Z0 ,

ZL=ZL* Z0=1.5 - j0.7 Ω*(100Ω) =150-j70 Ω

- Design an open-ended shunt-stub matching network to match a load ZL =70+j110 Ω to a 50 Ω impedance T-line. Chose the solution that minimizes the length of the through line.

- We normalize the load impedance:

ZL= 1.4+j2.2 Ω

- We found the normalized admittance by passing through the center of the circle and the intersection of the line and  is

- We moved along the perimeter clockwise going to the generator until we intersect the 1+j*b, where (b = 1.9)

- We found 0–j*b and draw a line that passes to center of the circle

- The intersection of the line and the outer circle of the smith chart gives us length and distance

- The length is found by subtracting from this distance 0.25 λ,

l=0.327 λ -0.25 λ=0.077 λ

- The distance that we need to travel is

d=0.185 λ+0.05 λ=0.19 λ

- A load impedance ZL=25+j90 Ω is to be matched to a 50 Ω line using a shorted shunt stub tuner. Find the solution that minimizes the length of the shorted stub

- We normalize the load impedance:

ZL= 0.5+j1.8 Ω

- We found the normalized admittance by passing through the center of the circle and the intersection of the line and  is

- We moved along the perimeter clockwise going to the generator until we intersect with the 1+ j*b, where (b=2.6)

- We found 0 – j*b and draw a line that passes to center of the circle;

- The intersection of the line and the outer circle of the smith chart gives us length and distance;

- The length is found by subtracting from this distance 0.25 λ,

l=0.308 λ -0.25 λ=0.058 λ

- The distance that we need to travel is

d=0.198 λ+0.078 λ=0.276 λ

- A 50 Ω  line is terminated in a pair of parallel load impedances of 50+j100 Ω and 50 –j100 Ω. Determine the total load admittance and impedance seen by the line.

- To find total load impedance we calculate

ZLT=50+j100*50-j10050+j100+50-j100=250 Ω

- Admittance is simply the inverse of impedance,

YL=1ZL=1250=0.004S

- To find normalized load we simply divide by 50 ,

ZL(T)50=25050=5 Ω

- Consider the figure with the following values VS=10V, RS=30Ω, R0=50Ω, up=0.666c, RL=150Ω, and l=10cm. Plot out to 2ns

- The voltage at the source end,

Γs=Rs-RoRs+Ro=30-5030+50=-0.25

ΓL=RL-RoRL+Ro=150-50150+50=0.5

V1+=R0R0+Rs*Vs=5050+30*10v=6.25v

V1-=V1+*ΓL=6.25v*0.5=3.125v

V2+=V1-*Γs=3.125v*-0.25=-0.781v