Experiment: #2:

- Basic Logic Gates

Marlon Myers

Digital Control

EMT1250

Professor: Prof. Patrick

Table of Contents:

| Objective | 3 |
| --- | --- |
| Equipment | 3 |
| Discussion | 3 |
| Schematic | 4 |
| Questions | 6 |
| Charts | 8 |
| Conclusion | 10 |

Objective:

To investigate the operation of the NOT, OR and AND Gates.

Equipment:

- Equipment/Parts Needed:

- 5V DC Power Supply

- Digital Trainer (Logic Probe)

- Breadboard

- DIP Switch

- 7404

- 7408

- 7432

Discussion:

- NOT, OR and AND gates are the basic logic gates. All other logic functions can be derived from these three.

- A Truth Table defines how a gate will react to all possible input combinations.

- A Logic Probe is a piece of test equipment which displays the logic level at a point in the circuit. 0 to 0.8V = Logic 0 and lights the L indicator. 2.0V to 5.0V = Logic 1 and lights the H indicator. Invalid logic voltage levels light either indicator.

Schematics:

Figure 2-1 Schematic for Not Gate

Figure 2-2 Schematic for Or Gate

Figure 2-3 Schematic for Or Gate

Questions:

- What are the Boolean expression for the Not, OR and gates?

Ans: AND = (x = AB)

OR = (x = A + B)

Not = (x = A)

- Under what input conditions is the output of a 2 input OR gate LOW? Under what input conditions is the output of a 2 input AND gate High?

Ans: a) When both inputs are low, or gate is low

b) When both inputs are high, and gate is high

- We are given a 3 input AND gate and want to use it as a 2 input AND gate. Two inputs to the AND gate are connected to the two signals. How would you connect the third gate input?

Ans: Connect third to VCC

- If one input to a 3 input OR gate were accidentally connected to VCC (shorted to VCC), how would the output of the OR gate react no matter the other 2 input levels might be?

Ans:  High

- If one input to a 3 input AND gate were accidentally connected to Ground (shorted to Ground), how would the output of the AND gate react no matter the other 2 input levels might be?

Ans: Low

Chart:

| Truth Table |  |  |  |
| --- | --- | --- | --- |
| A | X | A(volt) | X(volt) |
| 0 | 1 | 0 | 3.47 |
| 1 | 0 | 5.5 | 0.39 |

Figure 2-1 Schematic for Not Gate

| Truth Table |  |  | Volts Measured |  |  |
| --- | --- | --- | --- | --- | --- |
| A | B | X | A(volt) | B (volts) | X(volt) |
| 0 | 0 | 0 | 0 | 0 | 0.44 |
| 0 | 1 | 1 | 0 | 5.06 | 3.46 |
| 1 | 0 | 1 | 5.06 | 0 | 3.46 |
| 1 | 1 | 1 | 5.06 | 5.06 | 3.46 |

Figure 2-2 Schematic for Or Gate

| Truth Table |  |  | Volts Measured |  |  |
| --- | --- | --- | --- | --- | --- |
| A | B | X | A(volt) | B (volts) | X(volt) |
| 0 | 0 | 0 | 0 | 0 | 0.49 |
| 0 | 1 | 0 | 0 | 5.06 | 0.49 |
| 1 | 0 | 0 | 5.06 | 0 | 0.49 |
| 1 | 1 | 1 | 5.06 | 5.06 | 3.22 |

Figure 2-3 Schematic for Or Gate